|  | Simulation of Across Mount Date: Monday, June 16, 2014 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 4 Mesh Information 5 Study Results 7 Conclusion 10. |
| --- | --- |
| # Description June_16_2014 Panel Mount loaded from one side plate |  |

# Assumptions

# Model Information Model name: Across Mount Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. #8 (0.199) Diameter Hole2; Solid Body; Mass:1.42696 kg Volume:0.00018532 m^3 Density:7700 kg/m^3 Weight:13.9842 N; C:\Users\sfalcone\Desktop\GitHub\tinyPipes\mechanics\Panel Mount\sfalcone\Simple Mount\Across Mount.SLDPRT Jun 16 19:05:04 2014.

# Material Properties Model Reference; Properties; Components. Name:; Alloy Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 620.422 N/mm^2. Tensile strength:; 723.826 N/mm^2.; SolidBody 1(#8 (#8 (0.199) Diameter Hole2)(Across Mount).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 2450 N..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 5.61877 mm. Tolerance; 0.280939 mm. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 23903. Total Elements; 12139. Maximum Aspect Ratio; 6.0273. % of elements with Aspect Ratio < 3; 98.3. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:08. Computer name:; SFALCONE-THINK.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 2.00225e-007 N/mm^2 (MPa) Node: 10246; 307.067 N/mm^2 (MPa) Node: 1005. Across Mount-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 61; 1.9326 mm Node: 2982. Across Mount-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. Across Mount-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 2.02048 Node: 1005; 3.09863e+009 Node: 10246. Across Mount-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion